# VoIP Watcher — Extended Feature Proposals

Prepared:  April 14, 2026     Platform:  Sippy Softswitch / VoIP NOC Dashboard     Document:  Volume II — New Feature Proposals     App URL:  https://vo-ip-watcher--junaid70.replit.app

This document proposes 24 additional features not covered in the previous roadmap. They span real-time quality engineering, reseller operations, regulatory compliance, advanced analytics, and network diagnostics — all designed specifically for the Sippy Softswitch platform.

## 1. Network & Media Quality

#### 1. Jitter & Packet Loss Monitor  [Real-Time]

Tracks RTP stream quality metrics — jitter, packet loss %, and burst loss — per active call and per carrier. Complements existing MOS monitoring with lower-level media diagnostics.

Capture jitter and loss from Sippy RTP stats API per active call

Alert when carrier-level average jitter exceeds configurable threshold (e.g. >30 ms)

Historical jitter trend chart per vendor on the Graphs page

Color-coded jitter column added to the Live Calls Details table

Value:  Diagnoses network-layer quality problems that MOS alone cannot pinpoint.

#### 2. Codec Negotiation Analytics  [Analytics]

Tracks which audio codecs (G.711, G.729, Opus, G.722, etc.) are being negotiated and used across all calls, and flags excessive transcoding that adds latency and cost.

Parse codec fields from Sippy CDRs and live call data

Codec usage breakdown chart (pie/bar) on the Graphs page

Transcoding rate per carrier: flag vendors forcing costly transcode paths

Alert when a codec not on the approved list appears in live calls

Value:  Identifies inefficient codec paths and helps enforce codec policy across the network.

#### 3. DTMF Analytics & Failure Tracking  [Quality]

Monitors DTMF (touch-tone) signal success rates — critical for IVR systems and call centres.

Track DTMF method (RFC 2833, in-band, SIP INFO) per carrier from CDR metadata

Flag calls terminated with DTMF-related SIP cause codes

Alert when DTMF failure rate for a carrier exceeds threshold

Per-carrier DTMF reliability score in the Reports section

Value:  Prevents silent IVR failures that cause customer complaints without appearing in ASR stats.

#### 4. RTP Port & NAT Traversal Monitor  [Network]

Identifies RTP media flow failures caused by NAT or firewall issues — one of the most common causes of one-way audio in VoIP.

Monitor calls flagged by Sippy as having RTP timeout events

Group one-way-audio events by source IP range to identify NAT problem clusters

Alert when RTP timeout rate exceeds baseline for a specific client subnet

Suggested remediation steps shown inline (STUN, TURN, symmetric RTP)

Value:  Reduces 'dead air' complaints by identifying NAT/firewall issues before they escalate.

## 2. Routing & Cost Intelligence

#### 5. Least Cost Routing (LCR) Analyser  [Cost]

Analyses whether each completed call was routed via the cheapest available path, and flags cases where a lower-cost route was available but unused.

Cross-reference CDR destination prefix with all active vendor rate cards

Highlight CDRs where a cheaper route existed (savings shown in USD)

Monthly 'missed savings' report: total overpayment due to suboptimal routing

One-click 'optimise now' suggestion to reorder route priorities in Sippy

Value:  Directly reduces termination cost — often 5–15% savings on international traffic.

#### 6. Call Flow Simulator  [Engineering]

A sandbox tool that lets engineers test how a call will route — without placing a real call — by simulating the dialplan rules currently active on the switch.

Enter a source account + destination number to simulate

Step-by-step trace showing which route group, carrier, and rate card would be selected

Flag if no route is available (would result in 503) before the call is placed

Useful for pre-deployment testing of new dialplan rules

Value:  Eliminates 'trial-and-error' route testing that disrupts live traffic.

#### 7. Vendor SLA Scorecard  [Reporting]

Automated weekly scorecard ranking each termination vendor against contractual SLA targets for ASR, ACD, PDD, and MOS.

Configurable SLA targets per vendor (e.g. ASR > 65%, ACD > 90s)

Traffic light status per metric: Green / Amber / Red

Week-over-week trend for each vendor

Auto-email scorecard to vendor account manager every Monday

Value:  Creates accountability with carriers and provides evidence for SLA penalty claims.

#### 8. Cost Optimisation Recommendations Engine  [AI-Assisted]

Analyses 30 days of CDR data and generates ranked recommendations for routing improvements that would reduce cost or improve quality.

Rule-based engine (no external AI dependency required)

Example recommendations: 'Route Pakistan Mobile via Vendor B — 12% cheaper, same ASR'

Each recommendation shows estimated monthly saving and quality impact

One-click to accept: pushes route change to Sippy automatically

Value:  Provides actionable intelligence that typically takes hours of manual analysis to produce.

## 3. Advanced Fraud & Security

#### 9. Robocall Pattern Detector  [Fraud]

Identifies automated dialling campaigns by analysing statistical patterns in call timing, duration, and caller ID sequences — beyond what IRSF or FAS detection covers.

Detect sequential caller IDs (e.g. +1-555-0001 → 0002 → 0003)

Flag accounts with >200 calls/hour where average call duration <8 seconds

Identify 'neighbor spoofing': caller ID prefix matches dialled number area code

Block identified robocall source accounts automatically via Sippy XML-RPC

Value:  Protects carrier reputation and reduces STIR/SHAKEN attestation downgrades from regulators.

#### 10. SIP Brute-Force Login Monitor  [Security]

Detects SIP REGISTER brute-force attacks where attackers try many username/password combinations against SIP endpoints.

Track 401 Unauthorized responses per source IP over rolling 5-minute windows

Alert when a single IP generates >50 failed registrations in 5 minutes

Auto-add offending IP to a firewall blocklist via Sippy or system iptables API

Geographic origin of attack shown on the Traffic Map

Value:  Stops credential-stuffing attacks before they result in toll fraud.

#### 11. Concurrent Call Limit Enforcer  [Fraud]

Monitors whether individual accounts are exceeding their configured concurrent call limits and alerts or disconnects excess calls in real time.

Poll live calls per account every 10 seconds against the limit stored in Sippy

Alert NOC when any account exceeds its limit by >20%

Optional auto-disconnect: terminate the oldest excess call via XML-RPC

Limit breach history log per account

Value:  Prevents a single compromised account from monopolising switch capacity.

#### 12. Fraud Velocity Scoring  [Fraud]

Assigns a real-time fraud risk score (0–100) to each account based on multiple simultaneous behavioural signals — not just a single trigger.

Score components: destination risk, call velocity, CLI diversity, time-of-day anomaly, short-duration ratio

Score displayed live in the NOC Dashboard sidebar per account

Accounts above score threshold (e.g. 75) trigger an automated alert

Score history charted per account on the Client/Vendor detail page

Value:  Catches sophisticated fraud that evades single-signal detectors by combining multiple weak signals.

## 4. Client & Reseller Management

#### 13. Reseller White-Label Portal  [Business]

A separate, brandable portal that resellers can log into to view analytics, balances, and reports for only their assigned sub-accounts — without seeing other customers' data.

Reseller role (below Admin, above Viewer) with access scoped to assigned accounts only

Custom logo and colour scheme per reseller (white-label)

Reseller sees their own P&L: buy rate vs. sell rate margin

Reseller can download CDR reports and invoices for their sub-accounts

Value:  Enables the operator to offer a managed service to resellers without building a separate platform.

#### 14. SLA Monitoring & Breach Alerting  [Reporting]

Tracks actual service delivery against SLA commitments made to enterprise clients, with automatic breach notifications.

Define SLA per client: uptime %, ASR floor, max PDD, MOS floor

Real-time SLA compliance status per client on the Client detail page

SLA breach email sent to client contact and internal KAM immediately

Monthly SLA compliance PDF report generated automatically per client

Value:  Demonstrates service quality contractually and reduces dispute resolution time.

#### 15. Number Inventory Management  [Operations]

A comprehensive inventory of all number ranges owned or leased by the operator, beyond the basic DID list — with utilisation tracking.

Track number ranges by geography, provider, and lease cost

Utilisation rate: what % of each number range is assigned vs. available

Expiry tracking: alert 30 days before a number lease expires

Porting requests tracker: status of numbers being ported in/out

Value:  Prevents revenue loss from expired number leases and identifies underutilised inventory.

#### 16. Click-to-Call & Test Call Launcher  [Operations]

Launch test calls directly from the VoIP Watcher dashboard to verify route quality without switching to another tool.

Enter source account + destination number and click 'Place Test Call'

Call is placed via Sippy XML-RPC originate command

Real-time display of call progress (ringing, answered, duration)

CDR for the test call auto-linked and highlighted in the CDR Viewer

Value:  Dramatically speeds up route verification — no need for a physical SIP phone or softphone.

## 5. Advanced Analytics & Reporting

#### 17. Traffic Forecasting Dashboard  [Analytics]

Uses historical CDR data to predict next-day and next-week traffic volumes, helping NOC teams prepare for expected peaks.

Analyse same-day-of-week patterns over the past 8 weeks

Generate hourly traffic forecast for the next 24 hours

Confidence bands shown on chart (optimistic / expected / pessimistic)

Alert if actual traffic deviates >30% from forecast (unexpected drop or surge)

Value:  Improves NOC readiness; enables proactive capacity and staffing decisions.

#### 18. Geographic Revenue Heatmap  [Analytics]

A world map visualisation showing where revenue is generated and where costs are concentrated, broken down by destination country.

Colour intensity = revenue generated per country (from rate card × CDR minutes)

Toggle to view: Revenue / Cost / Margin / Call Volume

Click a country to drill into that country's CDRs, carrier breakdown, and ASR trend

Exportable as PNG for management presentations

Value:  Makes global traffic and revenue distribution immediately visible to non-technical management.

#### 19. Peer Carrier Benchmarking  [Analytics]

Compares the operator's traffic metrics against anonymised industry benchmarks for similar traffic profiles.

Benchmark ASR, ACD, PDD, and MOS against industry medians for each destination region

Traffic light indicator: performing above / at / below industry average

Benchmark data sourced from aggregated (anonymised) historical data within the platform

Monthly benchmark report delivered by email

Value:  Gives management a reference point for whether network quality is competitive.

#### 20. CDR Anomaly Detector (Statistical)  [Analytics]

Automatically flags CDR records that are statistically unusual compared to the baseline for that account, destination, and time window — a catch-all for unexplained changes.

Run nightly on the previous day's CDRs

Flag records where duration, cost, or destination distribution deviates >3σ from baseline

Anomaly inbox on the Reports page with severity score per flagged batch

Each anomaly shows the expected vs. actual value and a suggested investigation step

Value:  Acts as a safety net for fraud and routing issues that don't match known patterns.

## 6. Operations & Engineering Tools

#### 21. SIP Registration Database & Viewer  [Operations]

A real-time searchable database of all currently registered SIP endpoints pulled from Sippy, with device fingerprinting.

List all active SIP registrations: username, IP, user-agent, registration expiry

Search by username, IP, or user-agent string

Flag registrations from unexpected geographies or unusual user-agent strings

One-click force de-registration of a specific endpoint via Sippy XML-RPC

Value:  Gives NOC engineers full visibility into what is registered on the switch at any moment.

#### 22. Switch Configuration Change Tracker  [Engineering]

Expands the existing Sippy change-detection alerts into a full audit timeline showing exactly what changed, when, and what it looked like before vs. after.

Store before/after snapshots of Sippy account, route, and dialplan configs

Timeline view: scroll through all configuration changes in reverse chronological order

Diff view: side-by-side highlight of what changed in each config snapshot

Roll-back button: restore a previous configuration snapshot to Sippy via XML-RPC

Value:  Eliminates 'what changed?' guesswork during incident response.

#### 23. Maintenance Window Scheduler  [Operations]

Schedule planned maintenance windows that suppress alerts and adjust dashboards automatically — preventing alert fatigue during known outages.

Create maintenance windows with start/end time, affected components, and reason

Alerts for affected switches or hosts are suppressed during the window

Dashboard shows 'Maintenance in Progress' banner for affected entities

Post-maintenance report: metrics before, during, and after the window

Value:  Prevents NOC teams from being flooded with false alerts during planned work.

#### 24. Multi-Switch Consolidated View  [Engineering]

A single unified dashboard that aggregates live metrics across multiple Sippy switch instances simultaneously — for operators running more than one softswitch.

Add multiple Sippy switch connections in Settings

Consolidated NOC Dashboard: total live calls, ASR, and alarms across all switches

Per-switch drill-down: click any switch to view its isolated metrics

Cross-switch call correlation: identify calls handed between switches

Value:  Essential for operators scaling beyond a single switch or running geo-redundant deployments.

## Priority Summary

| Priority | Feature | Effort | Business Value |
| --- | --- | --- | --- |
| Critical | Robocall Pattern Detector (#9) | Medium | Very High |
| Critical | SIP Brute-Force Login Monitor (#10) | Medium | Very High |
| Critical | Fraud Velocity Scoring (#12) | High | Very High |
| High | LCR Analyser (#5) | Medium | Very High |
| High | Vendor SLA Scorecard (#7) | Low | High |
| High | Jitter & Packet Loss Monitor (#1) | Medium | High |
| High | Switch Config Change Tracker (#22) | Medium | High |
| High | SLA Monitoring & Breach Alerting (#14) | Medium | High |
| Medium | Concurrent Call Limit Enforcer (#11) | Low | High |
| Medium | Codec Negotiation Analytics (#2) | Low | Medium |
| Medium | Cost Optimisation Engine (#8) | High | Very High |
| Medium | Traffic Forecasting Dashboard (#17) | Medium | High |
| Medium | CDR Anomaly Detector (#20) | Medium | High |
| Medium | SIP Registration Database (#21) | Low | Medium |
| Medium | Maintenance Window Scheduler (#23) | Low | Medium |
| Nice to Have | Call Flow Simulator (#6) | Medium | Medium |
| Nice to Have | Reseller White-Label Portal (#13) | High | High |
| Nice to Have | Click-to-Call Launcher (#16) | Low | Medium |
| Nice to Have | Geographic Revenue Heatmap (#18) | Medium | Medium |
| Nice to Have | Multi-Switch Consolidated View (#24) | High | High |
| Nice to Have | Number Inventory Management (#15) | Medium | Medium |
| Nice to Have | DTMF Analytics (#3) | Low | Medium |
| Nice to Have | Peer Carrier Benchmarking (#19) | High | Medium |
| Nice to Have | RTP/NAT Traversal Monitor (#4) | Medium | Medium |

## Implementation Notes

All 24 features in this document are separate from and additive to the 24 features in Volume I.

Features #9, #10, #11, and #12 form a 'Fraud Intelligence Layer II' cluster — best built together after Volume I Tier 1.

Features #5 and #8 require the Carrier Rate Card Management feature from Volume I as a prerequisite.

Feature #24 (Multi-Switch) requires schema changes to support multiple switch connections — plan carefully.

Feature #13 (Reseller Portal) is the largest single feature — estimate 3–4 weeks of development.

Features #1 and #2 (Jitter, Codec) depend on Sippy exposing RTP and codec fields in its CDR API — verify availability first.

Feature #22 (Config Tracker) extends the existing Sippy change-detection system already in production.

All other features are buildable without external dependencies beyond the existing Sippy XML-RPC API.

VoIP Watcher Extended Feature Proposals — Volume II — April 14, 2026  |  Confidential